Documentation to FLAT-model

�������������������������������

We all know that the Rral-mode of the Intel'80xxx serious only allows a

memory-allocation of max. 1Mbyte.

If we want to use more memory we had to switch into the Protected-mode.

In that mode we are able to use the whole installed memory of the PC, but well

the Protected-mode is organized in a totally other way as the Real-mode, also

the CPU waste 10% more of the time to organize that Protected-mode compared

to the Real-mode.

Well, some time ago some fine hackers find a way to use the whole memory also

in the Real-mode. The trick is very simple, just swith into the Protected-mode

turn all segment-register to a limit of 4Gbyte and switch back to the Real-

mode without resetting the processor and well, we can know reach the whole

memory from the Real-mode.

In FLAT.INC i followed that way and included some routines to work with this

model.

The thing is easy, i switch into the Protected-mode, there i change the

segment-register ES,FS,GS to a limit of 4Gbyte and switch back to Real-mode.

Mostly i use the GS-register for getting into the memroy up to 1Mbyte.

But be careful you've to keep some things in mind.

You can't set the GS-register to 1Mbyte, you have to do it with the offset.

Example: We want to write a 0 into the 1Mbyte ram-position of the PC.

xor ax,ax     ; ax=0

mov gs,ax     ; gs=0

-now the GS-register is pointed to the beginn of the PC-ram, now we've

to add a 1Mbyte offset to it

mov esi,1024*1024

mov gs:esi,0

-now we've written a 0 at the 1Mbyte ram-position.

If you using a HIMEM-driver you've to add a free block of memory to the

offset, coz the HIMEM-driver loads programms into the HMA, that's the 1st

64Kbyte after the 1Mbyte-ram.

So we have to change out routine in following way:

xor ax,ax

mov gs,ax

mov esi,1024*1024+65535

mov gs:esi,0

that's it.......

Well, the routine in the FLAT.INC file always have those 1024*1024+65525

addition included.

So each time when you read or write to the memory about 1Mbyte over the GS-

register you alwas have those value added to the offset.

That means that when you give to the routine the offset 0 it means in real

1024*1024+65535.....

Totally simple.......