VIDTIMER reads and writes data to your video card, through the CPU window

located at address A0000. The data transfer is timed as "frames". A "frame"

is the time it takes for the raster beam of your monitor to scan from top

to bottom. In normal MCGA mode, the refresh rate of your monitor is set to

70 Hz. This means that 70 "frames" pass per second. This relationship,

together with the CPU clock speed, is used to calculate the various

statistics that appear on the screen when the program is run.

A typical run may look like this:

VIDTIMER v1.10a - A MAGE Utility - (C) Fabian Gonzalez, 1994

1024 x 16384 doublewords = 64 megabytes were written to video memory

Timing results: 184 frames, 2.63 seconds

Megabytes per second    :    24.35

Kilobytes per frame     :   356.00

Frames per 64 kilobytes :     0.18

CPU clocks per write    :    10.34

1024 x 4096 doublewords = 16 megabytes were read from video memory

Timing results: 149 frames, 2.13 seconds

Megabytes per second    :     7.52

Kilobytes per frame     :   109.00

Frames per 64 kilobytes :     0.15

CPU clocks per read     :    33.49

The number of frames used in transferring data is shown, together with its

equivalent in seconds. The throughput is shown in megabytes per second (how

much data can be transferred to/from video memory in a second), as kilobytes

per frame (the amount of data that can be transferred in a single frame) and

as frames per 64 KB. Frames per 64 KB shows the number of frames that pass in

the amount of time it takes to transfer 64 KB to/from video memory. The last

entry shows the CPU clockcyles used to transfer one doubleword (4 bytes)

to/from video memory.

Reads from video memory generally are a lot slower than writes. This is the

reason for the reduced data transfer amount in reads contra writes.

VIDTIMER accepts the following options:

-h         Displays the help screen

-c[speed]  Sets the CPU speed. This setting is used to calculate the number

of CPU clock cycles required per write/read. This setting defaults

to 66 MHz.

-w[frames] Shows full timing statistics for writes, based on the number of

frames used. This option is useful for getting the full statistics

from video cards listed in VIDCARDS.TXT.

-r[frames] Shows full timing statistics or reads, based on the number of

frames used. This option is useful for getting the full statistics

from video cards listed in VIDCARDS.TXT.